IC TRAINING CENTER VIET NAM

FUNDAMENTAL IC DESIGN AND VERIFICATION COURSE

FINAL PROJECT

TIMER IP DESIGN SPECIFICATION

Student:  Thong Nguyen

Class: IC39

Instructor: Tri Le

Contents

LIST OF TABLES	3

LIST OF FIGURES	3

1.Overview	4

1.1. Introduction	4

1.2. Main features	4

1.3. Block diagram	6

1.4. Interface signals	7

2.Register Specification	8

2.1. Register Summary	8

2.2. Timer Control Register (TCR)	8

2.3. Timer Data Register 0 (TDR 0)	10

2.4. Timer Data Register 1 (TDR 1)	10

2.5. Timer Compare Register 0 (TCMP 0)	11

2.6. Timer Compare Register 1 (TCMP 1)	11

2.7. Timer Interrupt Enable Register (TIER)	12

2.8. Timer Interrupt Status Register (TISR)	13

2.9. Timer Halt Control Status Register (THCSR)	14

3. Functional Description	15

3.1. APB slave	15

3.2. Register	17

3.2.1TCR	17

3.2.3 TDR0 and TDR1	18

3.2.4 TCMP0 and TCMP1	19

3.2.5TISR	19

3.2.6TIER	20

3.2.7THCSR	21

3.3.6 Read data circuit	22

3.3. Count control	22

3.4. Counter	23

3.5.Interrupt Control	24

4. History	25

# LIST OF TABLES

Table 1: Interface signals of timer IP 	7

Table 2: Register Summary	9

Table 3. Timer Control Register (TCR)	10

Table 4. Timer Data Register 0 (TDR0)	11

Table 5. Timer Data Register 1 (TDR1)	12

Table 6. Timer Compare Register 0 (TDR0)	12

Table 7. Timer Compare Register 1 (TCMP1)	13

Table 8. Timer Interrupt Enable  Register (TIER)	13

Table 9. Timer Interrupt Status Register (TISR)	14

Table 10. Timer Halt Control Status Register (THCSR)	15

Table 11. Update history	27

# LIST OF FIGURES

Figure 1: Block diagram of IP	6

Figure 2: Write access with wait state	16

Figure 3: Schematic for APB module	17

Figure 4: Schematic of TCR	19

Figure 5: Schematic of TDR0/1	20

Figure 6: Schematic of TCMP0/1	21

Figure 7: Schematic of TISR	22

Figure 8: Schematic of TIER	22

Figure 9: Schematic of THCSR	23

Figure 10: Schematic of read data circuit	24

Figure 11: Schematic of Count Control module	24

Figure 12: Example waveform of different count speed	25

Figure 13: Schematic of Adder module	26

Figure 14: Schematic of  Interrupt Control	26

# 1.Overview

## 1.1. Introduction

Timers are an essential module of every chip, they are used to generate accurate timing intervals or control the timing of various operations within the circuit. Timer can be used in various applications: pulse generation, delay generation, event generation, PWM generation, Interrupt generation ….

This timer module is customized from the CLINT module of industrial RISC-V architecture. It is used to generate interrupt based on users’ settings

## 1.2. Main features

- 64-bit count up

- 12-bit address

- Register set is configured via APB bus (IP is APB slave).

- Support wait state (1 cycle is enough) and error handling

- Support byte access o Support halt (stop) in debug mode

- Timer uses active low async reset.

- Counter can be counted based on the system clock or divided up to 256.

- Support timer interrupt (can be enabled or disabled).

Figure 1: block diagram of Timer IP

## 1.3. Block diagram

Figure 1: Block diagram of IP

APB slave

This sub-block processes the data from the APB bus in transfer between APB master and the IP’s register in read and write access to the register.

Register

This sub-block stores data of the IP, including interrupt data, counting data, and data that is written from APB bus

Counter control

Control between different counting speed, process data from register to configure counting speed of the IP, control between different counting modes.

Interrupt control

This sub-block control interrupt data and send to output of the IP and to the register.

Counter

This sub-block is the counter of the IP, which increments the count data and send back to the register.

## 1.4. Interface signals

Table 1: Interface signals of timer IP

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| sys_clk | 1 | Input | Clock. sys_clk is a clock signal. All APB signals are timed against the rising edge of sys_clk. |
| sys_rst_n | 1 | Input | Reset. sys_rst_n is the reset signal and is active-LOW. sys_rst_n is normally connected directly to the system bus reset signal |
| tim_psel | 1 | Input | Select. The Requester generates a tim_psel signal for each Completer. tim_psel indicates that the Completer is selected and that a data transfer is required. |
| tim_pwrite | 1 | Input | Direction. tim_pwrite indicates an APB write access when HIGH and an APB read access when LOW. |
| tim_penable | 1 | Input | Enable. tim_penable indicates the second and subsequent cycles of an APB transfer |
| tim_paddr[11:0] | 12 | Input | Address. tim_paddr is the APB address bus. |
| tim_pwdata[31:0] | 32 | Input | Write data. The tim_pwdata write data bus is driven by the APB bridge Requester during write cycles when tim_pwrite is HIGH. |
| tim_prdata[31:0] | 32 | Output | Read data. The tim_prdata read data bus is driven by the selected Completer during read cycles when PWRITE is LOW. |
| tim_pstrb[3:0] | 4 | Input | Write strobe. tim_pstrb indicates which byte lanes to update during a write transfer. There is one write strobe for each 8 bits of the write data bus. tim_pstrb[n] corresponds to tim_pwdata[(8n + 7):(8n)]. tim_pstrb must not be active during a read transfer. |
| tim_pready | 1 | Output | Ready. tim_pready is used to extend an APB transfer by the Completer. |
| tim_pslverr | 1 | Output | Transfer error. tim_pslverr is an optional signal that can be asserted HIGH by the Completer to indicate an error condition on an APB transfer. |
| tim_int | 1 | Output | Interrupt. tim_int is the output signal that flags HIGH by the Completer when the interrupt conditions is met. |
| dbg_mode | 1 | Input | Debug. dgb_mode is an input signal that can be asserted HIGH by the Requester when the user want to turn on halt mode. |

# 2.Register Specification

## 2.1. Register Summary

Table 2: Register Summary

| Offset | Abbreviation | Register name |
| --- | --- | --- |
| 0x00 | TCR | Timer Control Register |
| 0x04 | TDR0 | Timer Data Register 0 |
| 0x08 | TDR1 | Timer Data Register 1 |
| 0x0C | TCMP0 | Timer Compare Register 0 |
| 0x10 | TCMP1 | Timer Compare Register 1 |
| 0x14 | TIER | Timer Interrupt Enable Register |
| 0x18 | TISR | Timer Interrupt Status Register |
| 0x1C | THCSR | Timer Halt Control Status Register |
| Others | Reserved |  |

## 2.2. Timer Control Register (TCR)

Offset address: 0x0

Reset value: 0x0000_0100

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  | DIV_VAL[3:0] |  |  |  | Reserved |  |  |  |  |  | DIV_ EN | TIM_ EN |
|  |  |  |  | rw |  |  |  |  |  |  |  |  |  | rw | rw |

Table 3. Timer Control Register (TCR)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:12 | Reserved | - | 20’h0 | Reserved |
| 11:8 | DIV_VAL[3:0] | RW | 4’b0001 | Counter control mode setting: - 4’b0000: Counting speed is not divided - 4’b0001: Counting speed is divided by 2 (default) - 4’b0010: Counting speed is divided by 4 - 4’b0011: Counting speed is divided by 8 - 4’b0100: Counting speed is divided by 16 - 4’b0101: Counting speed is divided by 32 - 4’b0110: Counting speed is divided by 64 - 4’b0111: Counting speed is divided by 128 - 4’b1000: Counting speed is divided by 256 - Others: reserved (*)prohibit settings. When setting the prohibited value, div_val is not changed. Note: user must not change div_val while timer_en is High (*): div_val is prohibited to change when timer_en is High. Access is an error response in this case. (*)Access is “error response” when setting prohibit value to div_val |
| 7:2 | Reserved | RO | 6’b0 | Reserved |
| 1 | DIV_EN | RW | 1’b0 | Counter control mode enabling. - 0: Disabled. Counter counts with normal speed based on system clock - 1: Enabled. The counting speed of counter is controlled based on div_val Note: user must not change div_en while timer_en is High div_en is prohibited to change when timer_en is High. Access is error response in this case. |
| 0 | TIM_EN | RW | 1’b0 | Timer enable - 0: Disabled. Counter does not count. - 1: Enabled. Counter starts count Timer from High to Low will initialize TDR0/1 to their initial value |

## 2.3. Timer Data Register 0 (TDR 0)

Offset address: 0x4

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. Timer Data Register 0 (TDR0)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TDR0 | RW | 32’h0 | Lower 32-bit of the 64-bit counter. Value of this register is cleared to initial value when timer_en changes from High to Low |

## 2.4. Timer Data Register 1 (TDR 1)

Offset address: 0x8

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5. Timer Data Register 1 (TDR1)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TDR0 | RW | 32’h0 | Upper 32-bit of the 64-bit counter. Value of this register is cleared to initial value when timer_en changes from High to Low |

## 2.5. Timer Compare Register 0 (TCMP 0)

Offset address: 0x0C

Reset value: 32’hFFFF_FFFF

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCMP 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCMP 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6. Timer Compare Register 0 (TDR0)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TCMP0 | RW | 32’hFFFF_FFFF | Lower 32-bit of the 64-bit compare value. Interrupt pending bit is asserted when counter value is equal to compare value. |

## 2.6. Timer Compare Register 1 (TCMP 1)

Offset address: 0x10

Reset value: 32’hFFFF_FFFF

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCMP 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCMP 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7. Timer Compare Register 1 (TCMP1)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TCMP1 | RW | 32’hFFFF_FFFF | Upper 32-bit of the 64-bit compare value. Interrupt pending bit is asserted when counter value is equal to compare value. |

## 2.7. Timer Interrupt Enable Register (TIER)

Offset address: 0x14

Reset value: 32’h0

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  | int_en |

Table 8. Timer Interrupt Enable  Register (TIER)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:1 | Reserved | - | 31’h0 | Reserved |
| 0 | INT_EN | RW | 1’b0 | Timer interrupt enable - 0: Timer interrupt is disabled. - 1: Timer interrupt is enabled. When this bit is 0, no timer interrupt is output. When this bit is 1, timer interrupt pending bit can be output (interrupt pending bit is set when counter value is equals to compare value). Clearing this bit to 0 while interrupt is asserting will mask the interrupt to 0 but does not affect the interrupt pending bit (TISR.int_st bit). |

## 2.8. Timer Interrupt Status Register (TISR)

Offset address: 0x18

Reset value: 32’h0

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  | int_st |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | rw1c |

Table 9. Timer Interrupt Status Register (TISR)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:1 | Reserved | - | 31’h0 | Reserved |
| 0 | INT_ST | RW1C | 1’b0 | Timer interrupt trigger condition status bit (interrupt pending bit) - 0: the interrupt trigger condition does not occur. - 1: the interrupt trigger condition occurred. Write 1 when this bit is 1 to clear it to 0. Write 0 when this bit is 1 has no effect. Write to this bit when it is 0 has no effect. Note1: When the interrupt trigger condition occurs (counter reached compare value), counter continues to count normally. Note2: the clearance condition has the highest priority. |

## 2.9. Timer Halt Control Status Register (THCSR)

Offset address: 0x1C

Reset value: 32’h0

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  | halt_ack | halt_req |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ro | rw |

Table 10. Timer Halt Control Status Register (THCSR)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:2 | Reserved | - | 3’h0 | Reserved |
| 1 | HALT_ACK | RO | 1’b0 | Timer halt acknowledge - 0: timer is NOT halted - 1: timer is halted 1’b0 Timer accepts the halt request only in debug mode, indicates by debug_mode input signal |
| 0 | HALT_REQ | RW |  | Timer halt request 0: no halt req. 1: timer is requested to halt. |

# 3. Functional Description

## 3.1. APB slave

The timer uses an APB slave to receive data from the user to configure the timer. The user can write data to the timer’s register to turn on and off the timer or setting the counting speed, and the compare threshold for the timer.

Features of APB slave

- 12-bit width address

- 32-bit width read/write data

- Support wait state and error response

Figure 2: Write access with wait state

The APB protocol takes at least 2 clock cycles to write data into the IP’s register

- In the first cycle - the SET UP state, PENABLE set low, PSEL is set high, PADDR and PDATA is set

- In the second cycle, PENABLE and PWRITE are set high. From this cycle, the APB master waits for the high PREADY from the IP’s APB slave. When PREADY is high,  wr_en is set high, data is transferred to the register, and the data transmission ends

- In this case, the wait state is 1 clock cycle; thus in the third clock cycle, PREADY is set high,  wr_en is set high

- After the third clock cycle, PSEL is set low while other signals are “don’t care”, marking the end of one data transmission

Figure 3: Schematic for APB module

- wr_en is the signal sent to the register module, when wr_en is High, data is written to the register. wr_en is created as an AND of the three signals PSEL, PENABLE, PWRITE, which indicates the ACCESS state of APB interface.

- rd_en is the signal sent to the register module, when rd_en is High, data is read from the register. rd_en is created as an AND of the three signals PSEL, PENABLE, PWRITE, which indicates the ACCESS state of the APB interface.

- The registers are used to delay 1 clock cycle for the wait state so that wr_en or rd_en and PREADY it set high 1 cycle after  PENABLE and PWRITE is set high

- PREADY is the OR of wr_en and rd_en because PREADY is set high in the ACCESS state

Error response:

- Whenever an error happen in a write access, APB slave raise the PSLVERR signal and preventing new invalid data be written into the register

- Error response in APB signal is returned whenever one of the three cases happen:

  - Changing the value of div_val while counting

  - Changing the value of div_en while counting

  - Changing the value of div_val to prohibited value

Partial Data Transfer

- The APB slave supports partial data transfer, allowing users to write into each byte of the 32-bit registers

## 3.2. Register

#### 3.2.1TCR

Figure 4: Schematic of TCR

TCR is where the configuration of the timer is saved

- tim_en turns the counter on or off

- div_en decides the counting mode of the timer

- div_val decides the counting speed of the timer

Error response

- div_val only accepts certain values written into the register. If any invalid div_val value is written into the register, the register stays the same and keeps the previous value.

- div_en and div_val can only be written to when the timer is off. When data is written to div_en and div_val, the IP raises PSLVERR signal and keeps the previous data in the register.

#### 3.2.3 TDR0 and TDR1

Figure 5: Schematic of TDR0/1

- User can write into this register to configure starting point of counting

- The counter take value from TDR0 and TDR1 to count up

- After each increment, the counter save the new value back to TDR0 and TDR1

- TDR0 and TDR1 are reset to default value when the timer is turned off

#### 3.2.4 TCMP0 and TCMP1

Figure 6: Schematic of TCMP0/1

- Store the threshold value that the IP compares the count value in TDR to

- User can write into this register to configure the interrupt threshold of the IP

#### 3.2.5TISR

Figure 7: Schematic of TISR

TISR is where the timer save interrupt status from the interrupt controller

- Whenever TDR0 = TCMP0 and TDR1 = TCMP1, tim_int is set pending high and the status is saved to the TISR.tim_st

- tim_st is only reset when 1 is written to the bit.

- Write 0 will not have any affect on int_st

- Write 1 to int_st when the status is 0 will not change the data in the  register

- The reset function have the highest priority

#### 3.2.6TIER

Figure 8: Schematic of TIER

- TIER.int_en is the mask for tim_int signal output from the IP.

- tim_int is set pending high when TDR0 = TCMP0 and TDR1 = TCMP1. When this happens, if

- int_en = 1, tim_int = 1

- int_en = 0, tim_int = 0

#### 3.2.7THCSR

Figure 9: Schematic of THCSR

- Halt_reg is set by writing into THCSR to request for halting.

- Halting only happens when the IP’s input dbg_mode is set.

- When halting happens, halt_ack is set.

#### 3.3.6 Read data circuit

Figure 10: Schematic of read data circuit

- Which register is being read is decided by a mux with 9 inputs. Which input is connected to output is determined by the address input from APB slave.

## 3.3. Count control

Figure 11: Schematic of Count Control module

Functionality

- To control the counting speed, the IP’s counter control have an internal counter which decides how many cycles does it take for a the IP’s clock to increment by 1

When TCR.div_en is 1

- This internal counter counts to a limit determined by TCR.div_val. When the limit is reached, the counter control submodule send a count_en pulse to the IP’s counter which increments the counter by 1

- The counting function of the internal counter is controlled by THCSR.halt_ack, when the IP is halted, the internal counter stops counting but does not reset. Thus, when halting is stopped, the internal counter continues to count from its current value, maintaining the counting process normally

Figure 12: Example waveform of different count speed

When TCR.div_en is 0

- count_en is connected to tim_en; thus whenever the timer is on, the counter control will send a 1 signal to the counter, making it count up every clock cycle.

## 3.4. Counter

Figure 13: Schematic of Adder module

- The counter takes value from TDR0 and TDR1, when the signal count_en from the count control submodule is 1, the counter increments the count value by 1 and writes back to TDR0 and TDR1.

## 3.5.Interrupt Control

Figure 14: Schematic of  Interrupt Control

- This sub-module masks the interrupt status with interrupt enable value from the TIER register to control if the interrupt signal is sent

# 4. History

Table 11. Update history

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 19/8/2026 | 1.0 | Newly created | Thong Nguyen |
| 4/9/2026 | 1.1 | First fully finishes version | Thong Nguyen |
| 19/9/2026 | 1.2 | Minor updates on formating | Thong Nguyen |